# Serial Peripheral Interface

Advanced Logic Design

## Team Members:

- Ahmed Alaa El-Sayed Arabi Zidan

- Ahmed Ata

- Menatalh Hossamalden

- Sarah El-Zayat

Supervised by:

- Dr. Ihab Talkhan

- Eng. Mustafa Mahmoud

- Eng. Yahya Zakaria

### Part I: Master Module

Figure 1: Master Prototype

#### Ports:

| Port Name | Type / Size | Description |
| --- | --- | --- |
| clk | Input | The synchronous clock generated from the Microcontroller feeding the master of SPI. |
| reset | Input | Resets the Register’s Data to 0 (Asynchronous reset). |
| start | Input | The bit which trigger the transmission and receiving process. |
| slaveSelect | Input [1:0] | The number of the slave to be connected to. |
| masterDataToSend | Input [7:0] | The 8-bits data we put in the Register to be sent in the transmission process. |
| MISO | Input | The bit received by the master from slave at each clock pulse. |
| masterDataRecieved | Output reg [7:0] | The 8-bits data which The Slave Receives at the end of receiving process. |
| SCLK | output | The Synchronous clock of The Master. |
| CS | Output [0:2] | Enable bit of The Transmission (Active-Low). |

| MOSI | Output reg | The bit transmitted to the slave from the master at each clock pulse. |
| --- | --- | --- |

- Local and Unported Signals:

| Name | Type / Size | Description |
| --- | --- | --- |
| counter | integer | A counter that counts the number of bets |
| flag | reg | A bit raised to high when the transmission and receiving process are terminated. |
| buffer | Reg [7:0] | - The Register inside the master - A normal shift Register with MISO as its IL and MOSI as its IR. |

- Processes and The Data Flow:

  - At the raising edge of start:

    - The master read the slaveSelect.

    - Sent the CS bit for each slave (in the same time)

    - Initialize the masterDataRecieved by unknowns.

    - Initialize the buffer by masterDataToSend.

    - Initialize the counter by 1

    - Initialize the flag by zero

  - At the rising edge of SCLK (shifting process):

    - Shifting the LSB in the buffer into the MISO.

  - At the falling edge of SCLK (Sampling process):

    - Shifting the MOSI bit into the Register from the left.

    - Updating the SlaveDataReceived to contain the same bits as the Register.

  - At the raising edge of flag:

    - Both transmission and receiving process are terminated.

### PART II: Self-Checking Testbench for the Master Module

#### Local and Unported Signals:

| Name | Type | Description |
| --- | --- | --- |
| CLK | reg | Input to Master |
| Start | reg | Input to Master |
| Slave Select | reg [1:0] | Input to Master |
| reset | reg | Input to Master |
| MISO | reg | Input to Master |
| SCLK | wire | Output from Master |
| CS | wire [0:2] | Output from Master |
| MOSI | wire | Output from Master |
| masterDataToSend | reg [7:0] | Input to Master |
| masterDataReceived | wire [7:0] | output from master module |
| ExpectedSlaveDataToReceive | reg [7:0] | The 8-bits data which the Slave is expected to receive at the end of transmission |
| testcase_SlaveDataToSend | wire [7:0] array [1:4] | The 4-element array of 8-bit data which is expected to be sent by Master in transmission |
| testcase_MasterDataToSend | wire [7:0] array [1:4] | The 4-element array of 8-bit data which is an input to Master in transmission (masterDataToSend) |

- Processes and The Data Flow:

  - Initializing Local and unported signals:

    - Initialize start with 0 then 1 (triggering the start signal to start transmission).

    - Initialize input data loaded to Master and expected data to receive.

    - Initialize slave select.

  - At the rising edge of CLK (mimicking shifting process of Slave):

    - Setting MISO with 𝑖th  bit of testcase_SlaveDataToSend.

    - Increasing i by 1.

  - At the falling edge of CLK (sampling process):

    - Updating the ExpectedSlaveDataToReceive to contain all the bits shifted by Master and sampled by Slave.

#### Wave of Master simulation

Figure 2 : Wave form

D- Transcript

Figure 3 : Transcript

### Part III: Slave Module

Figure 4 Slave

#### Ports:

| Port Name | Type / Size | Description |
| --- | --- | --- |
| reset | Input | Resets the Register’s Data to 0 (Asynchronous reset). |
| slaveDataToSend | Input [7:0] | The 8-bits data we put in the Register to send in the transmission process. |
| slaveDataReceived | output reg [7:0] | The 8-bits data which The Slave Receives at the end of transmission. |
| SCLK | Input | The Synchronous clock of The Master. |
| CS | Input | Enable bit of The Transmission (Active-Low). |
| MOSI | Input | The bit received by slave from the master at each clock pulse. |
| MISO | Output reg | The bit transmitted to the master from slave at each clock pulse. |

- Local and Unported Signals:

| Name | Type / Size | Description |
| --- | --- | --- |
| Reg Data | reg [7:0] | - The Register inside The Slave - A normal shift Register with MOSI as its IL and MISO as its IR. |

- Processes and The Data Flow:

  - At the falling edge of CS:

    - Indicates the start of the transmission.

    - The Slave loads the data that will be transmitted into the register.

  - At the rising edge of CS:

    - Indicates the end of the transmission

    - Setting MISO to high-z.

  - At the rising edge of SCLK (shifting process):

    - Shifting the LSB in the register into the MISO.

  - At the falling edge of SCLK (Sampling process):

    - Shifting the MOSI bit into the Register from the left.

    - Updating the SlaveDataReceived to contain the same bits as the Register.

### Part IV: Self-Checking Testbench for the Slave Module

#### Local and Unported Signals:

| Name | Type | Description |
| --- | --- | --- |
| reset | reg | Input to Slave |
| SCLK | reg | Input to Slave |
| CS | reg | Input to Slave |
| MOSI | reg | Input to Slave |
| MISO | wire | output from Slave |
| slaveDataToSend | reg [7:0] | Input to Slave |
| slaveSataReceived | wire [7:0] | output from slave module |
| ExpectedMasterDataToReceive | reg [7:0] | The 8-bits data which the Master is expected to receive at the end of transmission |
| testcase_MasterDataToSend | wire [7:0] array [1:4] | The 4-element array of 8-bit data which is expected to be sent by Master in transmission |
| testcase_SlaveDataToSend | wire [7:0] array [1:4] | The 4-element array of 8-bit data which is an input to Slave in transmission (slaveDataToSend) |

- Processes and The Data Flow:

  - Initializing Local and unported signals:

    - Initialize CS with 1 then 0 (triggering falling edge of CS).

    - Initialize input data loaded to Slave.

  - Set CS =1:

    - Triggering rising edge of CS.

    - Checking if data received and sent by Slave is as Expected and display an indicative message.

  - At the rising edge of SCLK (mimicking the shifting process of Master):

    - Setting MOSI with 𝑖th  bit of testcase_MasterDataToSend.

    - Increase i by 1.

  - At the falling edge of SCLK (sampling process):

- Updating the ExpectedMasterDataToReceive to contain all the bits shifted by Slave and sampled by Master.

Figure 5: Wave form

Simulation of Slave:

Figure 6: Slave Displayed Messages

D.Simulation of Development TestBench:

Figure 7 : Development Wave Form

Figure 8 : Development Displayed Message